DATABASE Design and DEFINITIONS - R.Hess - 4/17/04

The fields and data design below will be added to the existing Oil and Gas Well Database for the purpose of completing the CO2 project.

CO2 reservoir rock  sandstone, conglomerate, sand, or gravel

Seal rock shale, mudstone, claystone, mud, clay, halite, gypsum, salt, or non-welded (possibly clay- or zeolite- altered) ash-flow tuff.

99999 (five nines) = NO DATA

PT - pre-Tertiary, TT - Tertiary, R or RES - Reservoir, S - Seal, STH - Single Thickest, P - Potential, B or BOT - Base of; or Bottom,  TH - Thickness

NEITHER A CO2 RESERVOIR ROCK NOR SEAL.

limestone, dolomite, fractured volcanic rock, fractured sandstone, quartzite, metamorphic rocks, or granite or other igneous rocks

Data to be collected from well records, if available, in wells within areas not otherwise excluded for consideration of CO2

Depth Measurements are in feet from top of well.  *** To be recorded irregardless of CO2 consideration.***

===============================================================

1. Total depth of well. ***

TD_1			N	6	Feet

2. Are there potential CO2 reservoir rocks in the well below 1 km (3281 ft) depth? If no, go to next well. ***

P_CO2_RES		C	1	Y/N

3. Is there a potential seal below 1 km and above that reservoir rock? If no, go to next well.

P_SEAL		C	1	Y/N

4. Depth to base of Cenozoic/Tertiary volcanic rocks and alluvium. ***

TT_BOT		N	5	Feet	***

AL_BOT - Alluvial Bottom Only	N	5 	Feet	***

5. Depth to base of deepest reservoir rock in pre-Tertiary sedimentary package.

RES_PT_DP5		N 	5	Feet

6. How fresh is the water in this deepest reservoir rock? (Total dissolved solids - TDS?) SEE - PTWQPP

7. How porous is this deepest reservoir rock? % of porosity? SEE - PTWQPP

8. How permeable is this deepest reservoir rock? K in millidarcy? SEE - PTWQPP

9. Thickness of the thickest single pre-Tertiary reservoir rock.

PT_R_STH		N 	5	Feet

10. How fresh is the water in this thickest pre-Tertiary reservoir rock? SEE - PTWQPP

11. How porous is this thickest pre-Tertiary reservoir rock? SEE - PTWQPP

12. How permeable is this thickest pre-Tertiary reservoir rock? SEE - PTWQPP

13. Total thickness of all pre-Tertiary reservoir rocks.

PT_R_TOT		N	5	Feet

14. Thickness of the thickest single pre-Tertiary seal rock above the deepest reservoir rocks.

PT_SE_STH		N	5	Feet

15. Total thickness of all pre-Tertiary seal rocks above the deepest reservoir rocks.

PT_SE_TOT		N	5	Feet

--16. Dept to base of deepest reservoir rock in Tertiary sedimentary package below 1 km.

TT_R_D_B		N	5	Feet

17. How fresh is the water in this deepest reservoir rock in Tertiary package? SEE - TTWQPP

18. How porous is this deepest reservoir rock in Tertiary package? SEE - TTWQPP

19. How permeable is this deepest reservoir rock in Tertiary package? SEE - TTWQPP

20. Thickness of the thickest single Tertiary reservoir rock below 1 km.

TT_R_STH		N	5	Feet

21. How fresh is the water in this thickest single Tertiary reservoir? SEE - TTWQPP

22. How porous is this thickest single Tertiary reservoir? SEE - TTWQPP

23. How permeable is this thickest single Tertiary reservoir? SEE - TTWQPP

24. Total thickness of all Tertiary reservoir rocks below 1 km.

TT_R_TOT		N	5	Feet

25. Thickness of thickest single Tertiary seal rock below 1 km.

TT_SE_STH		N	5	Feet

26. Total thickness of all Tertiary seal rocks below 1 km.

TT_SE_TOT		N	5	Feet

27. Total thickness of all Tertiary seal rocks below 1 km and above shallowest reservoir rock.

TT_SE_ABOV	N	5	Feet

28. Thickness of halite beds below 1 km.

HALITE		N	5	Feet

============== PTWQPP ========================

PRE_TERTIARY RESERVOIR Water Quality, Porosity, Permeability field:

PTWQPP	C	200

Questions 6, 7, 8, and 10, 11, 12

Data in this field will follow this format: Groups to be separated by a colon: Single items to be separated by a semi-colon;

PT_D: (pre-Tertiary deepest reservoir) in feet,

R (reservoir), Q (water freshness in TDS),  POR (% porosity), PER (Permeability in K millidarcy)

PT_TH: (pre-Tertiary Thickest reservoir) in feet,

R (reservoir), Q (water freshness in TDS),  POR (% porosity), PER (Permeability in K millidarcy)

Example - PT_D 3000; RQ 1500; RPOR 3%; RPER 200: PT_TH 2800; RQ 2000; RPOR 5%; RPER 300:

========================== TTWQPP ========================

TERTIARY RESERVOIR Water Quality, Porosity, Permeability field:

TTWQPP	C	200

Questions 17, 18, 19, and 21, 22, 23

Data in this field will follow this format: Groups to be separated by a colon: Single items to be separated by a semi-colon;

TT_D: (Tertiary deepest reservoir) in feet,

R (reservoir), Q (water freshness in TDS),  POR (% porosity), PER (Permeability in K millidarcy)

TT_TH: (Tertiary Thickest reservoir) in feet,

R (reservoir), Q (water freshness in TDS),  POR (% porosity), PER (Permeability in K millidarcy)

Example - TT_D 3000; RQ 1500; RPOR 3%; RPER 200: TT_TH 2800; RQ 2000; RPOR 5%; RPER 300:

=======================================================

FACTORS THAT CAN NOW BE DERIVED FROM THESE NUMBERS

Total thickness of potential reservoir rocks = #13 + #24

Total thickness of potential seal rocks above the deepest reservoir rock and below 1 km = #15 + #26

Reservoir rock to seal rock ratio = #A/#B, ~ sand/shale ratio

========= *** ADDITIONAL OIL & GAS INFORMATION *** =========

Top of Sheep Pass or Elko Formation: ***

SP_ELKO		N	5	Feet

Thickness of Sheep Pass or Elko Formation: ***

SP_ELKO_TH	N	5	Feet

Top of Chainman Shale Formation (Upper most call):  ***

CHAINMAN		N	5	Feet

Thickness of Chainman Shale Formation (Upper most call):  ***

CHAIN_TH		N	5	Feet

Top of Joana Limestone (Upper most call):  ***

JOANA_LS		N	5	Feet

Thickness of Joana Limestone (Upper most call):  ***

JOANA_TH		N	5	Feet

========================================================